"""

Racing Line Analyzer

Compares driver's racing line against optimal reference line

"""

import numpy as np

from scipy.spatial import distance

from typing import Dict, List

class RacingLineAnalyzer:

"""Analyzes racing line deviations from optimal path"""

def __init__(self, driver_data, reference_data):

self.driver_data = driver_data

self.reference_data = reference_data

def calculate_line_deviation(self):

"""Calculate deviation from reference line at each point"""

driver_x = self.driver_data.data['x_position']

driver_y = self.driver_data.data['y_position']

ref_x = self.reference_data.data['x_position']

ref_y = self.reference_data.data['y_position']

# Calculate point-by-point Euclidean distance

deviations = np.sqrt((driver_x - ref_x)**2 + (driver_y - ref_y)**2)

return deviations

def identify_corner_regions(self):

"""Identify corner regions based on lateral G-forces"""

lat_g = np.abs(self.reference_data.data['lat_g'])

distances = self.reference_data.data['distance']

# Corners are where lateral G exceeds threshold

corner_threshold = 0.5  # G's

in_corner = lat_g > corner_threshold

corners = []

corner_start = None

for i, is_corner in enumerate(in_corner):

if is_corner and corner_start is None:

corner_start = i

elif not is_corner and corner_start is not None:

corners.append({

'start_idx': corner_start,

'end_idx': i,

'start_dist': distances[corner_start],

'end_dist': distances[i],

'apex_idx': corner_start + np.argmax(lat_g[corner_start:i])

})

corner_start = None

return corners

def analyze_corner_performance(self, corner, deviations):

"""Analyze performance through a specific corner"""

start_idx = corner['start_idx']

end_idx = corner['end_idx']

apex_idx = corner['apex_idx']

# Calculate metrics

avg_deviation = np.mean(deviations[start_idx:end_idx])

max_deviation = np.max(deviations[start_idx:end_idx])

apex_deviation = deviations[apex_idx]

# Estimate time loss (simplified: deviation correlates with time loss)

# Assumption: 1m deviation ≈ 0.01s loss

time_loss = avg_deviation * 0.01

# Determine severity

if avg_deviation > 3:

severity = 10

elif avg_deviation > 2:

severity = 7

elif avg_deviation > 1:

severity = 5

else:

severity = 2

return {

'avg_deviation': avg_deviation,

'max_deviation': max_deviation,

'apex_deviation': apex_deviation,

'time_loss': time_loss,

'severity': severity

}

def analyze(self):

"""Run complete racing line analysis"""

# Calculate deviations

deviations = self.calculate_line_deviation()

# Identify corners

corners = self.identify_corner_regions()

# Analyze each corner

problem_corners = []

for i, corner in enumerate(corners):

corner_analysis = self.analyze_corner_performance(corner, deviations)

# Add corner identification

corner_name = f"Corner {i+1}"

if i < 8:  # Match with predefined corners

corner_name = ['Turn 1 (Copse)', 'Turn 2 (Maggots)', 'Turn 3 (Becketts)',

'Turn 4 (Chapel)', 'Turn 5 (Stowe)', 'Turn 6 (Vale)',

'Turn 7 (Club)', 'Turn 8 (Abbey)'][i]

corner_info = {

'name': corner_name,

'distance': corner['start_dist'],

**corner_analysis

}

# Flag significant issues

if corner_analysis['avg_deviation'] > 1.5:

problem_corners.append(corner_info)

# Sort problem corners by severity

problem_corners.sort(key=lambda x: x['severity'], reverse=True)

return {

'deviations': deviations,

'avg_deviation': np.mean(deviations),

'max_deviation': np.max(deviations),

'corners': corners,

'problem_corners': problem_corners,

'total_time_loss': sum(c['time_loss'] for c in problem_corners)

}

def get_entry_apex_exit_analysis(self, corner):

"""Detailed analysis of corner entry, apex, and exit"""

start_idx = corner['start_idx']

apex_idx = corner['apex_idx']

end_idx = corner['end_idx']

# Split corner into three phases

entry_end = start_idx + (apex_idx - start_idx) // 2

exit_start = apex_idx + (end_idx - apex_idx) // 2

deviations = self.calculate_line_deviation()

return {

'entry': {

'avg_deviation': np.mean(deviations[start_idx:entry_end]),

'phase': 'ENTRY'

},

'apex': {

'deviation': deviations[apex_idx],

'phase': 'APEX'

},

'exit': {

'avg_deviation': np.mean(deviations[exit_start:end_idx]),

'phase': 'EXIT'

}

}