"""

Braking Analyzer

Analyzes braking points, pressure, and acceleration zones

"""

import numpy as np

from typing import Dict, List, Tuple

class BrakingAnalyzer:

"""Analyzes braking performance compared to reference"""

def __init__(self, driver_data, reference_data):

self.driver_data = driver_data

self.reference_data = reference_data

self.brake_threshold = 10  # Minimum brake pressure to count as braking

def detect_braking_zones(self, data):

"""Detect braking zones in telemetry data"""

brake = data.data['brake']

distance = data.data['distance']

speed = data.data['speed']

zones = []

in_braking = False

zone_start = None

for i in range(len(brake)):

if brake[i] > self.brake_threshold and not in_braking:

# Start of braking zone

in_braking = True

zone_start = i

elif brake[i] <= self.brake_threshold and in_braking:

# End of braking zone

in_braking = False

if zone_start is not None:

zones.append({

'start_idx': zone_start,

'end_idx': i,

'start_distance': distance[zone_start],

'end_distance': distance[i],

'entry_speed': speed[zone_start],

'exit_speed': speed[i],

'max_brake': np.max(brake[zone_start:i]),

'avg_brake': np.mean(brake[zone_start:i])

})

return zones

def compare_braking_points(self, driver_zone, ref_zone):

"""Compare braking points between driver and reference"""

point_difference = driver_zone['start_distance'] - ref_zone['start_distance']

pressure_difference = driver_zone['max_brake'] - ref_zone['max_brake']

# Classify the issue

if point_difference > 5:

issue = "braking_too_early"

recommendation = f"Delay braking by {abs(point_difference):.1f}m"

elif point_difference < -5:

issue = "braking_too_late"

recommendation = f"Brake {abs(point_difference):.1f}m earlier"

else:

issue = "good_timing"

recommendation = "Braking point is optimal"

if pressure_difference < -10:

pressure_issue = "insufficient_pressure"

pressure_rec = f"Apply {abs(pressure_difference):.0f}% more brake pressure"

elif pressure_difference > 10:

pressure_issue = "over_braking"

pressure_rec = f"Reduce brake pressure by {abs(pressure_difference):.0f}%"

else:

pressure_issue = "good_pressure"

pressure_rec = "Brake pressure is optimal"

return {

'point_difference': point_difference,

'pressure_difference': pressure_difference,

'timing_issue': issue,

'pressure_issue': pressure_issue,

'timing_recommendation': recommendation,

'pressure_recommendation': pressure_rec

}

def match_braking_zones(self, driver_zones, ref_zones):

"""Match driver braking zones with reference zones"""

matched_zones = []

for i, ref_zone in enumerate(ref_zones):

# Find closest driver zone

ref_distance = ref_zone['start_distance']

closest_driver_zone = None

min_distance = float('inf')

for driver_zone in driver_zones:

dist_diff = abs(driver_zone['start_distance'] - ref_distance)

if dist_diff < min_distance:

min_distance = dist_diff

closest_driver_zone = driver_zone

if closest_driver_zone and min_distance < 200:  # Within 200m

comparison = self.compare_braking_points(closest_driver_zone, ref_zone)

matched_zones.append({

'turn_number': i + 1,

'driver_zone': closest_driver_zone,

'reference_zone': ref_zone,

'comparison': comparison,

**comparison

})

return matched_zones

def analyze_trail_braking(self, zone):

"""Analyze trail braking technique"""

start_idx = zone['start_idx']

end_idx = zone['end_idx']

brake = self.driver_data.data['brake'][start_idx:end_idx]

# Check if driver is releasing brake progressively (trail braking)

# or releasing abruptly

brake_gradient = np.diff(brake)

# Good trail braking has gradual release (negative gradient)

if len(brake_gradient) > 0:

avg_release_rate = np.mean(brake_gradient[brake_gradient < 0])

if avg_release_rate > -2:  # Very gradual

return "excellent_trail_braking"

elif avg_release_rate > -5:

return "good_trail_braking"

elif avg_release_rate > -10:

return "moderate_trail_braking"

else:

return "abrupt_release"

return "insufficient_data"

def analyze(self):

"""Run complete braking analysis"""

# Detect braking zones

driver_zones = self.detect_braking_zones(self.driver_data)

ref_zones = self.detect_braking_zones(self.reference_data)

# Match and compare zones

matched_zones = self.match_braking_zones(driver_zones, ref_zones)

# Calculate summary statistics

early_braking = sum(1 for z in matched_zones if z['timing_issue'] == 'braking_too_early')

late_braking = sum(1 for z in matched_zones if z['timing_issue'] == 'braking_too_late')

over_braking = sum(1 for z in matched_zones if z['pressure_issue'] == 'over_braking')

under_braking = sum(1 for z in matched_zones if z['pressure_issue'] == 'insufficient_pressure')

avg_point_difference = np.mean([z['point_difference'] for z in matched_zones]) if matched_zones else 0

avg_pressure_difference = np.mean([z['pressure_difference'] for z in matched_zones]) if matched_zones else 0

return {

'zones': matched_zones,

'summary': {

'total_zones': len(matched_zones),

'early_braking_count': early_braking,

'late_braking_count': late_braking,

'over_braking_count': over_braking,

'under_braking_count': under_braking,

'avg_point_difference': avg_point_difference,

'avg_pressure_difference': avg_pressure_difference

}

}

def calculate_braking_efficiency(self, zone):

"""Calculate braking efficiency score"""

entry_speed = zone['driver_zone']['entry_speed']

exit_speed = zone['driver_zone']['exit_speed']

brake_distance = zone['driver_zone']['end_distance'] - zone['driver_zone']['start_distance']

# Calculate deceleration

if brake_distance > 0:

deceleration = (entry_speed - exit_speed) / brake_distance

# Reference deceleration

ref_entry = zone['reference_zone']['entry_speed']

ref_exit = zone['reference_zone']['exit_speed']

ref_distance = zone['reference_zone']['end_distance'] - zone['reference_zone']['start_distance']

ref_deceleration = (ref_entry - ref_exit) / ref_distance if ref_distance > 0 else 0

# Efficiency as percentage of reference

if ref_deceleration > 0:

efficiency = (deceleration / ref_deceleration) * 100

return min(100, max(0, efficiency))

return 50  # Default moderate efficiency